AMPEX AUDIO COMPANY

INFORMATION BULLETIN

TURNTABLE PIN

In recent months there have been a number of complaints of turntable pins shearing off under unusual operating conditions on some of the earlier versions of Models 300, 350, and S-3200.

While it is possible to replace these three small pins, a better solution is to install a single large pin, which is now available through all Authorized Service Representatives in Kit #17952-1.  This Kit contains a Pin, a Nut, and a Template.

To install the Pin, remove the Turntable Motor Assembly from the Tape Transport. Slide the Template over the motor shaft and mark the hole location lightly with a centerpunch at the point indicated by the cross (11/16" radius on the turntable).  Wrap a cloth around the motor to prevent drill chips from falling into it.  Drill a #28 hole to clear the 6-32 threads of the Pin.

The Pin should fit snugly against the Turntable.  Therefore after drilling the hole, it is necessary to enlarge the hole in the Pad to accommodate the 3/16" dia. Pin.

The Pin can then be inserted in the Turntable and held firmly with the 6-32 Nut provided.